# HP Insight Manager WBEM System Fan Provider Overview

| Description | The HP Insight Management Web-Based Enterprise Management (WBEM) System Fan provider implements and extends CIM classes described in this document to model the base server. These classes provide computational information and capabilities and serve as an aggregation point to associate one or more elements that make up a base server, such as: Fans, PowerSupplies, Processor and Memory (Volatile and/or NonVolatile Storage). |  |  |
| --- | --- | --- | --- |
| Profile Name |  | Organization | Version |
| HP Fan Profile |  | HP WBEM TC | 1.3 (P102) |
| HP Location Profile |  | HP WBEM TC | 1.0 (P124) |
| HP Fan Physical Location Profile |  | HP WBEM TC | 1.0 (P115) |

| Requirements | For each hardware architecture listed, this provider requires the following distributions HP Integrity managed servers; SLES 10 and later RLEL 5.0 and later. HP ProLiant managed servers; SLES 11 and later RHEL 5.3 and later. |
| --- | --- |
| Release History | Initial release with HP Insight Management WBEM Providers for Linux v2.0. |

## Setting Up the Provider

| Installing the Provider | There are no special installation instructions for this provider. It is installed by default as part of the HP Insight Management WBEM providers. |
| --- | --- |
| Configuring the Provider | This provider does not accept specific configuration adjustments beyond standard HP Insight Management WBEM support. |

## Using the Provider

Namespaces Supported by the Provider This provider returns instances in the root/hpq namespace.

| Schema Supported by the Provider |  |
| --- | --- |
|  | This provider supports the following classes: - SMX_Fan - SMX_FanCollection - SMX_FanRedundancySet - SMX_MemberOfFanRedundancy - SMX_HostedFanRedundancySet - SMX_MemberOfFanCollection - SMX_HostedFanCollection - SMX_SystemFan - SMX_FanLocation - SMX_FanElementLocation - SMX_FanModule - SMX_SystemPhysicalFan - SMX_RealizesFan - SMX_FanSlot - SMX_FanInSlot - SMX_SystemFanSlot - SMX_FanSpeedSensor - SMX_AssociatedFanSensor - SMX_SystemFanSensor |

The tables in the following sections describe the properties of the supported classes. The classes are categorized by the class or superclass that defines the property, the first column is the Property Name (including type and units) and the second column describes how the provider determines the properties implementation. When the Property Implementation value is a number, the number given is the default behavior and the Managed Object Format interpretation is within parenthesis. If other values are returned, a problem is indicated.

Unless otherwise noted, all of the property implementation values given are for HP ProLiant and HP Integrity (cellular and non-cellular) systems. The location related properties and implementation values are determined based on the server type so they may differ.

### SMX_Fan Class

The SMX_Fan class is used to model fans on servers. Fan numbers for existing instances do not change when other fans are added and removed.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short description of this fan from the name displayed by the MP or iLO and the location. For example: Fan 1 at CabinetFan=01 |
| Description | Description of this fan, including ElementName, FanType and StatusDescription[0]. For example: elementname provides cooling for fantype. statusdescription. Where: <elementname> represents the value of ElementName, fantype represents the corresponding string value of the FanType enumeration and statusdescription represents the value of StatusDescriptions[0] |
| ElementName | Short description of this fan from the name displayed by the MP or iLO and the location. For example: Fan 1 at CabinetFan=01 |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | 2 (OK) – if fan is operating properly; 3 (Degraded) – if fan is degraded 6 (Error) – if fan has failed |
| StatusDescriptions[0] | Description of the status of the fan corresponding to the OperationalStatus. For example: Fan is operating properly– if OperationalStatus[0] is 2 (OK); Fan is degraded – if OperationalStatus[0] is 3 (Degraded) Fan has failed – if OperationalStatus[0] is 6 (Error) |
| HealthState | 5 (OK) – if fan is operating properly; 10 (Degraded) - if fan is degraded 20 (Major Failure) – if fan has failed |
| Name | Short description of this fan from the name displayed by the MP or iLO and the location. For example: Fan 1 at CabinetFan=01 |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_Fan |
| DeviceID | Unique identifier for this fan instance. |
| SystemCreationClassName | SMX_ComputerSystem.CreationClassName |
| SystemName | SMX_ComputerSystem.Name |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_CoolingDevice |  |
| ActiveCooling | TRUE |
| CIM_Fan |  |
| VariableSpeed | True – if fan supports variable speed False – if fan does not support variable speed or is unknown |
| HP_Fan |  |
| FanType | Enumeration describing the type of fan. Please refer to the Managed Object Format. |
| CurrentSpeed | Speed of the fan in CurrentSpeedUnits |
| CurrentSpeedUnits | Enumeration describing the units for CurrentSpeed. Please refer to the Managed Object Format. |
| CurrentSpeedUnits | Enumeration describing the units for CurrentSpeed. Please refer to the Managed Object Format. |

### SMX_FanCollection Class

The SMX_FanCollection class is used to model fan collections on servers. There is one instance of this class on systems modeling fans.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Cooling Collection |
| Description | This is a collection of all devices including fans that provide cooling. GroupStatusDescription Where: GroupStatusDescription represents the value of the GroupStatusDescriptions[0] property. |
| ElementName | Cooling Collection |
| CIM_SystemSpecificCollection |  |
| InstanceID | Unique identifier for this cooling collection instance. |
| HP_GroupSystemSpecificCollection |  |
| GroupOperationalStatus[0] | Represents the most severe cooling redundancy status and non-fault tolerant fan operational status: 0 (Unknown) – if fan collection status is unknown; 2 (OK) – if fans are operating properly; 3 (Degraded) – if no SMX_Fan.OperationalStatus[0] for all non-fault tolerant fans report 6 (Error) and SMX_FanRedundancySet.RedundancyStatus for any cooling redundancy sets report 3 (Degraded Redundancy) or 4 (Redundancy Lost) and no cooling redundancy sets report 5 (Overall Failure). 6 (Error) – if SMX_Fan.OperationalStatus[0] for any non-fault tolerant fans report 6 (Error) or SMX_FanRedundancySet.RedundancyStatus for any cooling redundancy sets report 5 (Overall Failure). Note: If the system starts up with a redundancy status set to 4 (Redundancy Lost) and no fans have been removed or failed, the GroupOperationalStatus will be set to 2 (OK) because this is a valid configuration. |
| GroupStatusDescriptions[0] | Unknown status for fans if GroupOperationalStatus[0] is 0 (Unknown); Fans are operating properly if GroupOperationalStatus[0] is 2 (OK); Fault tolerant fans are degraded if GroupOperationalStatus[0] is 3 (Degraded); Fans have failed if GroupOperationalStatus[0] is 6 (Error) |

### SMX_FanRedundancySet Class

The SMX_FanRedundancySet class is used to model fan redundancy sets on servers.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short description of this redundancy set, for example: Cooling Redundancy Set <n> Where: <n> represents the number of the redundancy set instance. |
| Description | Description of this redundancy set For example: Cooling Redundancy Set <n> is collection of fault tolerant fans that provide redundant cooling Where: <n> represents the number of the redundancy set instance. |
| ElementName | User printable name for this cooling redundancy set. For example: Cooling Redundancy Set <n> Where: <n> represents the number of the redundancy set instance. |
| CIM_SystemSpecificCollection |  |
| InstanceID | Unique identifier for this cooling redundancy instance. |
| CIM_RedundancySet |  |
| MaxNumberSupported | Represents the maximum number of fans supported for this redundancy set. |
| MinNumberNeeded | Represents the minimum number of fans needed to function. |
| TypeOfSet | 3 (Load Balanced) |
| LoadBalanceAlgorithm | 0 (Unknown) |
| RedundancyStatus | 0 (Unknown) – if the redundancy status is unknown; 2 (Fully Redundant) – if all fans in the set are operating properly and enough to achieve redundancy; 3 (Degraded Redundancy) – if there are at least enough fans for the redundancy set to provide cooling, but fans have failed; 4 (Redundancy Lost) – if there are not enough fans required to achieve redundancy, but enough for the redundancy set to provide cooling; 5 (Overall Failure) – if there are not enough fans operating properly for the redundancy set to provide cooling. |

### SMX_MemberOfFanRedundancy Class

The SMX_MemberOfFanRedundancy class is used to represent an association between the HP_Fan and HP_FanRedundancySet classes.  The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| Collection | References HP_FanRedundancySet |
| Member | References HP_Fan |

### SMX_HostedFanRedundancySet Class

The SMX_HostedFanRedundancySet class is used to represent an association between HP_FanRedundancySet and the computer system that contains this redundancy set.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| OwningElement | References HP_ComputerSystem |
| OwnedElement | References HP_FanRedundancySet |

### SMX_MemberOfFanCollection Class

The SMX_MemberOfFanCollection class is used to represent an association between the HP_Fan and HP_FanCollection classes.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| Collection | References HP_FanCollection |
| Member | References HP_Fan |

### SMX_HostedFanCollection Class

The SMX_HostedFanCollection class is used to represent an association between HP_FanCollection and the computer system that contains this collection.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | References HP_ComputerSystem |
| Dependent | References HP_FanCollection |

### SMX_SystemFan Class

The SMX_SystemFan class is used to represent an association between HP_Fan and the computer system that contains this fan.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | References HP_ComputerSystem |
| PartComponent | References HP_Fan |

### SMX_FanLocation Class

The SMX_FanLocation class is used to model fan locations on servers.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short description of this fan location, including the name displayed by the MP or iLO. For example on HP ProLiant: Fan 1 at CabinetFan=01 |
| Description | Description of this fan location, including the name displayed by the MP or iLO. For example on HP ProLiant: The location of the Fan 1. Please refer to the fan position CabinetFan=01 printed on the system. |
| ElementName | User displayable name of this fan location, including the name displayed by the MP or iLO. For example on HP ProLiant: Fan 1 at CabinetFan=01 |
| CIM_Location |  |
| Name | Unique identifier for this fan location instance. |
| PhysicalPosition | Opaque identifier for the physical position of this fan |
| HP_Location |  |
| ElementLocationTag | Location string based on ElementLocationTagDesc. For example, on HP ProLiant: CabinetFan=01 |
| ElementLocationTagDesc | On HP ProLiant: 0 (HoodTag) |
| LocationInfoDesc | Array of location descriptions: On ProLiant: 4 (Socket) |
| LocationInformation | Array of string descriptions of the location based on LocationInfoDesc |

### SMX_FanElementLocation Class

The SMX_FanElementLocation class is used to represent an association between the CIM_Location and CIM_ManagedElement classes.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| Element | References CIM_ManagedElement |
| PhysicalLocation | References CIM_Location |

### SMX_FanModule Class

The SMX_FanModule class is used to model physical fans on servers. Fan numbers for existing instances do not change when other fans are added and removed.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short description of this fan from the name displayed by the MP or iLO and location. For example: Fan 1 at CabinetFan=01 |
| Description | Description of this fan module, including ElementName, type and status. For example: <ElementName> provides cooling for FanType. <StatusDescription> Where: <ElementName> represents the value of ElementName, FanType represents the corresponding string value of the FanType enumeration and <StatusDescription> represents the value of StatusDescriptions[0]. |
| ElementName | Name of this fan from the name displayed by the MP or iLO and location. For example: Fan 1 at CabinetFan=01 |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | 2 (OK) – if fan is operating properly; 3 (Degraded) – if fan is degraded; 6 (Error) – if fan has failed |
| StatusDescriptions[0] | Description of the status of the fan corresponding to the OperationalStatus. For example: Fan is operating properly – if OperationalStatus[0] is 2 (OK); Fan is degraded – if OperationalStatus[0] is 3 (Degraded) Fan has failed – if OperationalStatus is 6 (Error) |
| HealthState | 5 (OK) – if fan is operating properly; 10 (Degraded) if fan is degraded 20 (Major Failure) – if fan has failed |
| Name | Name of this fan module. For example: Fan 1 at CabinetFan=01 |
| CIM_PhysicalElement |  |
| CreationClassName | SMX_FanModule |
| Tag | Unique identifier for this fan instance. |
| CIM_PhysicalPackage |  |
| RemovalConditions | 0 (Unknown) 3 (Removable when off) 4 (Removable when on or off) |
| PackageType | 7 (Fan) |

### SMX_SystemPhysicalFan Class

The SMX_SystemPhysicalFan class implements the HP_SystemPhysicalFan class and association the HP_ComputerSystemChassis instance with HP_FanModule.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | References HP_ComputerSystemChassis |
| PartComponent | References HP_FanModule |

### SMX_RealizesFan Class

The SMX_RealizesFan class implements the HP_RealizesFan class and associates the HP_Fan instance with HP_FanModule instances.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | References HP_FanModule |
| Dependent | References HP_Fan |

### SMX_FanSlot Class

The SMX_FanSlot class is used to model physical fan slots on servers.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short description of this fan slot, to include the number and location. For example: Fan 1 at CabinetFan=01 |
| Description | Description of the fan slot. For example: Fan 1 at CabinetFan=01 |
| ElementName | Name of the fan slot. For example: Fan 1 at CabinetFan=01 |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | 2 (OK) – if fan slot is operating properly; |
| StatusDescriptions[0] | Fan slot is operating properly – if OperationalStatus[0] is 2 (OK); |
| HealthState | 5 (OK) – if fan is operating properly; |
| Name | Name of the fan slot. For example: Fan 1 at CabinetFan=01 |
| CIM_PhysicalElement |  |
| CreationClassName | SMX_FanSlot |
| Tag | Unique identifier for this fan slot. |
| CIM_PhysicalPackage |  |
| ConnectorLayout | 6 (Slot) |
| Number | <n> Where: <n> represents the number of the fan slot. |

### SMX_SystemFanSlot Class

The SMX_SystemFanSlot class implements the HP_FanSlot class and association the HP_ComputerSystemChassis instance with HP_FanSlot.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | References HP_ComputerSystemChassis |
| PartComponent | References HP_FanSlot |

### SMX_FanInSlot Class

The SMX_FanInSlot class implements the HP_FanInSlot class and associates the HP_FanSlot instance with HP_FanModule instances.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | References HP_FanSlot |
| Dependent | References HP_FanModule |

### SMX_FanSpeedSensor

SMX_FanSpeedSensor implements CIM_NumericSensor and is used to model fan tachometer sensors on Servers.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short description of the fan speed sensor from the name displayed by the MP or iLO and the location. For example: Fan speed sensor for Fan 1 at CabinetFan=01 |
| Description | Description of the fan speed sensor including name displayed by the MP or iLO, the location, and status. For example: Fan 1 detects fan speed. Located at CabinetFan=01. Fan speed sensor is operating properly. |
| ElementName | Short description of the fan speed sensor from the name displayed by the MP or iLO and the location. For example: Fan speed sensor for Fan 1 at CabinetFan=01 |
| CIM_ManagedSystemElement |  |
| OperationalStatus[0] | 2 (OK) – if fan sensor is operating properly |
| StatusDescriptions[0] | Fan speed sensor is operating properly—if OperationalStatus[0] is 2 (OK) |
| HealthState | 5 (OK) – if fan sensor is operating properly |
| Name | Name of fan speed sensor from the name displayed by the MP or iLO and the location. For example: Fan speed sensor for Fan 1 at CabinetFan=01 |
| CIM_LogicalDevice |  |
| CreationClassName | SMX_FanSpeedSensor |
| DeviceID | Unique identifier for fan slot. |
| SystemCreationClassName | HP_ComputerSystem.CreationClassName |
| SystemName | HP_ComputerSystem.Name |
| CIM_EnabledLogicalElement |  |
| EnabledDefault | 2 (Enabled) |
| EnabledState | 5 (Not Applicable) |
| RequestedState | 12 (Not Applicable) |
| CIM_Sensor |  |
| SensorType | 5 (Tachometer) |
| CurrentState | Normal |
| PossibleStates | Array can contain any of the following values: PossibleStates[0] = ”Normal”; |
| CIM_NumericSensor |  |
| CurrentReading | Current reading of fan speed sensor |
| BaseUnits | 19 (RPM) 65 (Percentage) |
| RateUnits | 0 (None) |
| UnitModifier | 0 |

### SMX_SystemFanSensor

SMX_SystemFanSensor is used to represent an association between SMX_FanSensor and the computer system that contains this fan sensor.

| Property Name | Property Implementation |
| --- | --- |
| GroupComponent | References HP_ComputerSystem |
| PartComponent | References SMX_FanSensor |

### SMX_AssociatedFanSensor

SMX_AssociatedFanSensor is used to represent an association between HP_Fan and the SMX_FanSpeedSensor that is reporting the speed of the fan.

| Property Name | Property Implementation |
| --- | --- |
| Antecedent | References SMX_FanSpeedSensor |
| Dependent | References SMX_Fan |

## Provider Indications

| Indications Generated by the Provider | The following tables describe the SMX WBEM System Fan Provider indications that are implemented for HP ProLiant server platforms, where available. |
| --- | --- |

Note: HP ProLiant does not support Fan Degraded and Fan Redundancy Failed indications.

### Provider Indication Common Properties

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication. |
| IndicationTime | Time of indication. |
| CIM_AlertIndication |  |
| EventTime | Time of the event or time of the indication if event time unknown. |
| SystemName | SMX_ComputerSystem.Name |
| SystemCreationClassName | SMX_ComputerSystem.CreationClassName |
| HP_AlertIndication |  |
| ProviderVersion | Provider Version in the format VV.UU.FF. For example: 01.05.00 |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication. |
| OSType | On ESX, 39 (VM) On Linux, 36 (Linux) |
| OSVersion | The operating system version of the computer system generating the indication in the following format: major.minor.build |
| SystemFirmwareVersion | Array of firmware versions of the computer system generating the indication. |
| SystemSerialNumber | Serial number of the computer system generating the indication. |
| SystemProductID | Product ID of the computer system generating the indication. |
| SystemModel | Model name of the computer system generating the indication. |
| SystemGUID | Platform GUID of the computer system generating the indication. |
| SystemVirtualUUID[] | If Synergy is enabled, this will contain the logical UUID for the system. |
| SystemVirtualSerialNumber[] | If Synergy is enabled, this will contain the logical Serial Number for the system. |
| EnclosureName | HP_BladeEnclosureCS.Name |
| RackName | Rack name, if one exists |
| RackUUID | Rack Unique Identifier, if one exists |
| BladeName | HP_ComputerSystem.Name |
| BladeBay | HP_BladeCSLocation.LocationInformation[0] |

The following indications are implemented for Proliant server platforms where available. System Fan indications are described below.

### HP_DeviceIndication: Fan Removed

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 2 (Information) |
| CIM_AlertIndication |  |
| Description | A fan has been removed <Fan> Where: <Fan> is the fan number that was removed. |
| AlertingManagedElement | WBEM Path of the SMX_Fan instance. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 1 |
| ProviderName | HP Cooling |
| RecommendedActions | Check the fan configuration and ensure that this fan was removed intentionally. |
| HP_AlertIndication |  |
| Summary | Fan removed |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Fan Removed |

### HP_DeviceIndication: Fan Inserted

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 2 (Information) |
| CIM_AlertIndication |  |
| Description | A fan has been inserted. <FanCaption> Where: <FanCaption> is the SMX_Fan.Caption for the fan that has been inserted. |
| AlertingManagedElement | WBEM Path of SMX_Fan instance representing the inserted fan. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 2 |
| ProviderName | HP Cooling |
| RecommendedActions | No action is recommended. |
| HP_AlertIndication |  |
| Summary | Fan inserted |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Fan inserted |

### HP_DeviceIndication: Fan Failed

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 5 (Major) |
| CIM_AlertIndication |  |
| Description | A fan has failed. <FanCaptio> Where: <FanCaption> is the SMX_Fan.Caption for the fan that has failed. |
| AlertingManagedElement | WBEM Path of SMX_Fan instance representing the failed fan. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 3 |
| ProviderName | HP Cooling |
| RecommendedActions | Check the failed fan and replace if necessary. |
| HP_AlertIndication |  |
| Summary | Fan failed |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 94 (Fan Failure) |
| ProbableCauseDescription | Fan Failed |

### HP_DeviceIndication: Fan Repaired

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 2 (Information) |
| CIM_AlertIndication |  |
| Description | A fan has been repaired. The fan is now operating properly. <FanCaption> Where: <FanCaption> is the SMX_Fan.Caption for the fan that has been repaired. |
| AlertingManagedElement | WBEM Path of SMX_Fan instance representing the failed fan. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 4 |
| ProviderName | HP Cooling |
| RecommendedActions | No action is recommended. |
| HP_AlertIndication |  |
| Summary | Fan repaired |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Fan Repaired |

### HP_DeviceIndication: Fan Degraded

Important:

This indication is not supported on HP ProLiant systems because the fans do not report a degraded status.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 4 (Minor) |
| CIM_AlertIndication |  |
| Description | A fan has degraded. <FanCaption> Where: <FanCaption> is the SMX_Fan.Caption for the fan that has degraded. |
| AlertingManagedElement | WBEM Path of SMX_Fan instance representing the degraded fan. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 9 |
| ProviderName | HP Cooling |
| RecommendedActions | Check the degraded fan and replace if necessary. |
| HP_AlertIndication |  |
| Summary | Fan degraded |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Fan Degraded |

### HP_DeviceIndication: Fan Redundancy Reduce

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 2 (Information) |
| CIM_AlertIndication |  |
| Description | A fault tolerant fan is no longer providing cooling for a cooling redundancy set. <RedundancySetCaptio>) Where: <RedundancySetCaption> represents the SMX_FanRedundancySet.Caption for the redundancy set with reduced redundancy. |
| AlertingManagedElement | WBEM Path of SMX_FanRedundancySet instance representing the redundancy set with reduced redundancy. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 5 |
| ProviderName | HP Cooling |
| RecommendedActions | No action is recommended. |
| HP_AlertIndication |  |
| Summary | Cooling redundancy reduced |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 88 (Loss of Redundancy) |
| ProbableCauseDescription | Cooling Redundancy has been Reduced |

### DeviceIndication: Fan Redundancy Lost

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 4 (Minor) |
| CIM_AlertIndication |  |
| Description | Cooling redundancy has been lost. <RedundancySetCaptio>) Where: <RedundancySetCaption> represents the SMX_FanRedundancySet.Caption for the redundancy set that lost redundancy. |
| AlertingManagedElement | WBEM Path of SMX_FanRedundancySet instance representing the redundancy set where redundancy was lost. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 6 |
| ProviderName | HP Cooling |
| RecommendedActions | Check the fan configuration and check the status of the cooling redundancy. Ensure the system is being cooled adequately. Add or replace fans if necessary. |
| HP_AlertIndication |  |
| Summary | Cooling redundancy lost |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 88 (Loss of Redundancy) |
| ProbableCauseDescription | Cooling Redundancy has been Lost |

### HP_DeviceIndication: Fan Redundancy Restored

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 2 (Information) |
| CIM_AlertIndication |  |
| Description | Cooling redundancy for the system has been restored. (RedundancySetCaption) Where: RedundancySetCaption is SMX_FanRedundancySet.Caption of the cooling redundancy set with restored redundancy. |
| AlertingManagedElement | WBEM Path of SMX_FanRedundancySet instance representing the cooling redundancy set with restored redundancy. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 8 |
| ProviderName | HP Cooling |
| RecommendedActions | No action is recommended. |
| HP_AlertIndication |  |
| Summary | Cooling redundancy restored |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | Cooling Redundancy has been Restored |

### HP_DeviceIndication: Fan Redundancy Failed

Important:

This indication is not supported on HP ProLiant systems because redundancy set failure, which is failure beyond redundancy lost, will cause system shutdown.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 5 (Major) |
| CIM_AlertIndication |  |
| Description | Cooling redundancy for the system has failed. (RedundancySetCaption) Where: RedundancySetCaption is SMX_FanRedundancySet.Caption of the cooling redundancy set with failed redundancy. |
| AlertingManagedElement | WBEM Path of SMX_FanRedundancySet instance representing the cooling redundancy set with failed redundancy. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 5 (Device Alert) |
| EventID | 10 |
| ProviderName | HP Cooling |
| RecommendedActions | Check the fan configuration and check the status of the cooling redundancy. Ensure the system is being cooled adequately. Add or replace fans if necessary. |
| HP_AlertIndication |  |
| Summary | Cooling redundancy failed |
| EventCategory | 23 (System Cooling) |
| ProbableCause | 88 (Loss of Redundancy) |
| ProbableCauseDescription | Cooling Redundancy has Failed |